Smart Campus IoT Simulation and Analytics Platform
Project Proposal

Course: Internet of Things  |  Program: Master of IT
Supervisor: Dr. Ammar Zahary  |  Sana'a University

Team Members:
1. Osama Haider
2. Saba Al-wesabi

# 1. Problem

Modern campuses contain classrooms, labs, offices, and server rooms that need continuous monitoring for comfort, safety, and energy efficiency. Building a full hardware IoT prototype is costly and time-consuming for a course project. This project solves that by creating a simulation-only IoT platform that demonstrates the complete IoT workflow without physical sensors.

# 2. Scenario

Smart Campus Environment Monitoring with 7 virtual devices across a university campus:

| Device ID | Label | Building | Purpose |
| --- | --- | --- | --- |
| hall_1_1 | Hall 1.1 | floor_1 | Teaching environment monitoring |
| hall_1_2 | Hall 1.2 | floor_1 | Teaching environment monitoring |
| hall_1_3 | Hall 1.3 | floor_1 | Teaching environment monitoring |
| hall_1_4 | Hall 1.4 | floor_1 | Teaching environment monitoring |
| server_room | Server Room | it_building | High-temperature risk monitoring |
| lab1 | Lab 1 | engineering_building | Lab comfort & air quality |
| office1 | Office 1 | admin_building | Office energy & occupancy |

# 3. Selected Tools

| Tool | Purpose |
| --- | --- |
| Python 3 | Virtual device simulator and MQTT subscriber |
| Paho MQTT | Python MQTT client library |
| HiveMQ Broker | Public MQTT broker for testing |
| Node-RED | Dashboard for live visualization |
| python-docx | Report generation |

# 4. Wireless Communication Model

Selected model: IEEE 802.15.4 / Zigbee-like.

Justification: Campus room sensors send small packets at low frequency and may be battery-powered. Zigbee-like communication is ideal for short-range indoor sensor networks with low power requirements.

The simulator includes: packet loss rate, random delay, RSSI, link quality, estimated energy consumption, and battery drain per transmission.

# 5. System Architecture

Virtual Device Layer → Python devices generate simulated sensor readings.

Communication Layer → MQTT topics transfer telemetry from devices to subscribers.

Processing Layer → Python subscriber validates, computes derived values, and triggers alerts.

Presentation Layer → Node-RED Dashboard displays live values and trends.

# 6. MQTT Topic Design

campus/{building}/{room}/{deviceId}/telemetry

Example:

campus/floor_1/hall_1_1/hall_1_1/telemetry

# 7. Expected Deliverables

Runnable Python simulator (7 virtual devices).

MQTT subscriber with validation, analytics, and alerts.

Node-RED Dashboard flow JSON.

Test cases documentation.

Final technical report (8–12 pages).

Demo video (5–8 minutes).

Short presentation (5–7 slides).